| ［ 公司名稱 Company Name ］ 品質管理 — Quality Management | QTS-IUX2-ITR-000001 版次 Rev：— 狀態：IN PROGRESS |
| --- | --- |

檢驗及測試記錄 INSPECTION & TEST RECORD

| 編號 Reference No. | QTS-IUX2-ITR-000001 | 主旨 Subject | Level 3 Slab Pour Inspection |
| --- | --- | --- | --- |
| 檢驗日期 Inspection Date | 2026-09-29 | 版次 Version | — |
| 到期日 Due Date | — | 結案日期 Close-out Date | — |
| NOI 編號 NOI No. | QTS-IUX2-NOI-000001 | 承包商 Contractor | Meridian Concrete Works Ltd. |
| 關聯 ITP Related ITP | QTS-IUX2-ITP-000001 | NCR 編號 NCR No. | — |
| 狀態 Status | In Progress | 提出人 Raised By | — |

QTS-IUX2-CHK-000001 — Rebar Cover & Lap Splice Check（Ongoing）

| # | 項目 Item | 標準 Criteria | 現況 Situation | 結果 Result |
| --- | --- | --- | --- | --- |
| 1 | Rebar cover thickness | 40mm +/-5mm per S-301 Rev.C | Rebar cover checked at 8 locations along Grid C4-C7 using a calibrated cover meter (serial no. CM-2291, calibration valid until 2027-01-15); all readings between 42mm and 47mm, within the 40mm+/-5mm tolerance specified on Structural Drawing S-301 Rev.C. Lap splice lengths measured at 3 column-beam junctions (C4, C5, C6); all exceeded the required 40x bar diameter minimum. Minor honeycombing observed at the underside of the C5-C6 beam soffit near the formwork joint, approx. 15cm x 8cm, noted for patch repair before final finish inspection — does not affect structural cover or splice compliance. SITU-MARK-ONE-abc123 | 未填寫 Not filled |
| 2 | Formwork alignment | Plumb within 6mm per storey | UNTOUCHED-ITEM-MARK-keep-me | 未填寫 Not filled |

不符合地點  Found Location

Block C / Level 3 / Grid C4-C7

說明  Details/Description

Level 3 Slab Pour Inspection

| 1 製表 Prepared By 姓名：____________ 日期：____________ 單位 Company：____________ | 2 複核 Reviewed By 姓名：____________ 日期：____________ 單位 Company：____________ | 3 核准 Approved By 姓名：____________ 日期：____________ 單位 Company：____________ |
| --- | --- | --- |